LPKF 91S

Manual

July jj

# µ

1.	Introduction	4

2.	Generally	4

2.1	Scope of supply 	4

2.2	Optional accessories	4

2.3	Installation	4

2.4	Machine orientation	5

2.5	LPKF 91S displays and connections	6

2.6	What is BoardMaster for?	8

3.	Safety instructions	8

4.	Commissioning	8

4.1	Before switching on	8

4.2	Switching on procedure	8

4.3	After switching on	9

4.4	Function test with BoardMaster	9

4.5	HOME position and two-pin system	9

4.6	Programming the HOME position in BoardMaster	9

4.7	Making a new two-pin system	10

4.8	Checking the HOME position	11

5.	The LPKF 91S operating displays	11

6.	Computer-controlled functions of the LPKF 91S	11

6.1	Connection to a PC	11

6.2	The LPKF 91S command set	12

6.2.1	Command structure	12

6.2.2	HP-GL standard commands	13

6.2.3	Special commands	14

6.2.4	Direct commands	17

6.3	Tutorial on producing a double-sided PCB	17

7.	Tools and accessories	19

7.1	Tools	19

7.2	Pen holder (optional)	19

8.	Milling and drilling with the LPKF 91S	20

8.1	Securing the PCB on the machine bed	20

8.2	How the mechanical working depth limiter works	21

8.3	Functional elements on the mill/drill head	22

8.4	Drilling	23

8.5	Isolation milling	24

8.6	Contour milling in PCB material	24

8.7	Milling wide isolation channels	25

8.8	Front plate engraving	25

8.9	Milling layout films	25

8.9.1	Correction agents	26

8.9.2	Colouring milled films	26

8.9.3	Coating removal	26

8.10	Base material	27

8.11	Cleaning the PCB	27

8.12	Practical tips	28

9.	Appendix	30

9.1	Maintenance	30

9.2	Serial port SERIAL 1 	30

9.3	Serial port SERIAL 2 	31

9.4	Motor connection 	31

9.5	Mill head connection 	31

9.6	EPROMs 	31

9.7	Fuses 	32

9.8	List of LPKF tools available 	33

9.9	Spare parts for the LPKF 91S 	37

# 1.	Introduction

The LPKF 91S mill/drill unit is a circuit board plotter which can be used to produce prototype PCBs and gravure films, and for engraving aluminium or plastic.

Familiarity with the BoardMaster functions is essential for operation of the machine.  Operation of the LPKF 91S with the LPKF91S.EXE machine driver is described in the LPKF91S Driver Manual.

# 2.	Generally

## 2.1.	Scope of supply

1.	1 LPKF 91S machine unit with integrated electronics

2.	1 RS cable (LPKF 91S control unit - computer), AT adapter, 9/26 pole

3.	1 set of accessories (adhesive tape, div. Allen key, alignment pins, tweezers, brush, 30 mm steel pin, 7 mm box spanner, 2 red two-pin strips)

4.	1 mains cable

5.	this manual

## 2.2.	Optional accessories

The following accessories can be supplied for the LPKF 91S:

1.	Dust extractor with fine filter.  The fine filter is essential in particular when handling materials containing glass fibre, such as for FR4 base material.

2.	Machine table

3.	Noise and dust guard hood (from mid 1994)

4.	Pen holder.

## 2.3.	Installation

1.	Important:  the equipment must stand on a flat and firm base.

2.	Set up the circuit board plotter so that the connecting cables to the electronic unit can move freely.

3.	Connect the LPKF 91S to the computer with the RS cable (COM1 or COM2).

4.	Connect the LPKF 91S control unit to the mains.

5.	Fit the dust extractor on the adapter (suitable for stay tube no.111124 of a Nilfisk industrial vacuum cleaner).

6.	There is a transport safety fitting on the DC mill/drill motor bracket to prevent any damage to the unit during transport.  Please remove it.

## 2.4.	Machine orientation

µ §

LPKF 91S: the x-axis is the longer direction of movement

Description of the main machine positions:

1.	Pause position.  The machine will move to this point to feed or turn the material to be machined.

2.	Home position.  This is on the machine's mirror axis (two-pin system) and serves as a reference position inside the machine driver (BoardMaster and LPKF 91S).

3.	Two-pin slide with hole for alignment pin.

4.	Reference pin for front two-pin strip.

5.	Tool change position (zero position).  The tool is changed here.

## 2.5.	LPKF 91S displays and connections

µ §

Front view of LPKF 91S

Description of the front panel:

1.	Operating display

2.	Error display

3.	X motor

4.	DC motor

5.	Working depth limiter

µ §

Rear view of LPKF 91S

Description of the rear panel:

1.	Dust extractor connection

2.	Serial interface (9-pole) for expansions

3.	Serial interface (25-pole) for connection to PC

4.	X motor connection

5.	Y motor connection

6.	Mill/drill head connection

7.	On/off switch with mains connection

8.	X motor

9.	Y motor

10.	DC motor

11.	Lifting magnet

12.	Working depth limiter

## 2.6.	What is BoardMaster for?

The plot data generated by postprocessing is temporarily stored in HP-GL files where LPKF programs are used.  The BoardMaster's job is to read in these HP-GL files, decode the plot commands and adapt them for the machine used.  This is essential as, although all such equipment works in a similar way, different command sets are used.  The LPKF 91S circuit board plotter is HP-GL compatible, i.e. the format generated by the driver is HP-GL.  The drivers can make modifications, such as stretch, turn, copy etc.

All LPKF circuit board plotters are driven via an asynchronous interface (RS232C).  In the case of the LPKF 91S, it is operated at 9600 baud, 1 stop bit, 8 data bits, no parity and hardware handshake.  The serial interface of the computer must be configured in the same way, using MODE commands (see MS-DOS description), or must be set to operate with Windows.

# 3.	Safety instructions

	Never reach into the machine while it is running.

	Remember that the machine changes speed automatically during the process.

	Only change the tool when the mill/drill motor is at a standstill.

	Insert the tool into the clamping device as far as it will go.

	Where chemicals are used, please observe the instructions on the containers or any separate safety data sheets.

	If you modify the equipment yourself, the equipment's safety can no longer be guaranteed and no guarantee claims can be accepted.

	Always work with a dust extractor.

# 4.	Commissioning

## 4.1.	Before switching on

Before switching on, all items are to be removed from the machine and its range of movement.  Pilot pins must not project from the base material.  Either a tool or the 3 mm steel pin must always be in the mill/drill motor shaft and secured with the Allen screw, otherwise when the motor is switched on, the Allen screw can be ejected from the shaft.

Never reach into the machine while it is running.

## 4.2.	Switching on procedure

The following switching on order is useful, but not essential:

1.	computer

2.	LPKF 91S

3.	start the BoardMaster program.

## 4.3.	After switching on

When it has been switched on, the equipment moves to limit switches -x and -y, and the system then halts in the tool change position.  It then awaits commands from the machine drivers.  If the equipment does not move when it has been switched on, check the following:

1.	Is the POWER LED on?

2.	Are all cables properly inserted?

3.	If necessary, check the fuses in the mains connection.

## 4.4.	Function test with BoardMaster

First check that the baud rate for the serial interface used is set to 9600 baud.  Also check that Channel and the correct interface have been selected in BoardMaster under Machine Connect...  Consult the BoardMaster Manual if you need help.

Now, using BoardMaster:

1.	turn on the motor

2.	raise/lower the head

3.	switch off the motor

4.	move the head manually using the cursor keys on the BoardMaster function bar.

## 4.5.	HOME-Position and two-pin system

The HOME position must be on the machine's mirror axis (two-pin system) for double-sided plates.  The double-sided PCB is turned around this mirror axis.  Inaccuracies in the HOME position result in a shift when machining double-sided PCBs after turning.

Note:

Two-pin system parallelism is only guaranteed for the current position of the two-pin strip.  The front two-pin slide must be pushed against the pin in the two-pin groove.  If base material of a different size is used, an additional alignment hole must be drilled in the rear two-pin strip.  To do this, move from the home position in the direction of the two-pin strip, shift it to the required position and make an additional hole in the slide by hand with a 3 mm drill.  The slide position for the various holes should be noted or marked on the circuit board plotter, thus ensuring parallelism for different formats.

## 4.6.	Programming the HOME position in BoardMaster

The home position needs to be programmed on new equipment and when the system has been moved.

The HOME position of the LPKF 91S must be programmed as described above precisely on the mirror axis (the two-pin system with the red plastic slides) for machining double-sided PCBs.  To do this, proceed as follows:

1.	Start BoardMaster.

2.	Click Machine in the menu bar.

3.	Now select Settings...

4.	Activate Unlock so that you can make entries in this window.

5.	Check that SMCU is selected.  If there is no cross in the relevant box, the box must be clicked.

6.	Now Initialise must be clicked.  The equipment travels to all four end positions, and then stops in the ZERO position.  The movement range thus determined is displayed under Size and stored when the Machine Setting.. dialog box is left.  For more detailed information see the BoardMaster Manual.

7.	A two-pin system has already been set up in the works.  The machine stores the HOME position x and y coordinates.  They should now be entered under Home in BoardMaster.

8.	If the coordinates are not known, the two-pin system must be drilled again.  To do this, see the section entitled Making a new two-pin system.

9.	When the Machine Settings.. dialog box is closed and BoardMaster left, the new values are saved.

## 4.7.	Making a new two-pin system

The alignment holes in the red two-pin strip become larger over time due to use and are therefore not as accurate as initially.  You must then drill new holes in the slides.  If after a certain time there are too many holes in the slides, the two-pin strips must be changed.

To make a new two-pin system, proceed as follows:

1.	First press both two-pin strips into the machine groove, with the front two-pin strip pushed against the reference pin to the front of the groove.  The distance between the two two-pin strips should be about the size of the base material in the X axis.

2.	Fit a drill measuring 3 mm in diameter so that the distance between the base plate and the drill point is about 0.5 mm.  This means that, as an exception to the rule, it may not be inserted as far as it will go.

3.	Set the working depth limiter as far up as possible using the knurled screw, so that the drill lift is lengthened.

4.	Move the mill/drill head to HOME and then to the front two-pin strip.  Then manually drill a hole about 4 mm deep in the approximate centre by pressing down the drill head.  Afterwards, do not move the mill/drill head position in the Y axis.

5.	Move the mill/drill head about 25 mm in the +x direction and define this position as the new HOME position (Set Home).

6.	Move in the X axis to the second, rear two-pin strip and drill a hole about 4 mm deep there too.

7.	Now position the drill tool in the drill chuck as far as it will go.

8.	Move the mill/drill head to the side.  Insert two alignment pins in the holes now made in the two-pin strip.  Check that the pins are firmly inserted as any play affects alignment accuracy.

9.	Mark old holes with a felt pen so they cannot be confused with the new ones.

10.	Position the previously drilled base material and drilling base over the pass pins.  The base material format should be selected such that the two-pin slide does not have to be moved more than 10 mm, as otherwise the two-pin system becomes less accurate.

11.	Now fix the base material thus positioned with adhesive tape too (Tesakrepp 5250 recommended) on all four sides.

## 4.8.	Checking the HOME position

1.	Make alignment holes in double-sided base material.

2.	Fix the material and the drilling base (2 mm) on the base plate with the help of the alignment pins (see too the section on fixing the PCB on the machine bed).

3.	Drill a hole at the HOME position with a 0.7 mm drill.

4.	Move the mill/drill head to the PAUSE position.

5.	Turn the PCB material around the X axis.

6.	Move to the HOME position, move to the tool change position and insert a universal milling cutter.  When the tool has been changed, the mill/drill head automatically returns to the HOME position.

7.	Mill a channel over the hole manually in the X direction, without moving in the Y direction.

8.	Check visually that the milled channel passes precisely through the centre point of the hole.

9.	If the milled line does not run exactly through the centre point, the HOME position must be corrected by half the difference.  This can be done directly by changing the home position in the Machine Settings... dialog box.

10.	After making a correction, the movement should be repeated as a check.

# 5.	The LPKF 91S operating displays

1.	POWER (green):  control operating display.  This only comes on when the system has been initialised (limit switch reached).

2.	ERROR (red):  the error light comes on for the following errors:  1. unexpected triggering of the limit switch,  2. overstep of soft frame.  In both cases, the system is reinitialised and the light then goes out.  3.  Quick stop is triggered.  In this case, the LED has to be deleted by the REPEAT command.

# 6.	Computer-controlled functions of the LPKF 91S

## 6.1.	Connection to a PC

The LPKF 91S has two serial interfaces.  The first (SERIAL 1) is used for connection to the controlling computer (PC).  The second interface (SERIAL 2) is provided for future applications.

The LPKF 91S control unit's SERIAL 1 is connected to a serial interface on the computer with the RS cable supplied.

The RS232 cable (zero modem) for the LPKF 91S circuit board plotter is wired as follows:

25-pole port			25-pole port

PC (COMx)			circuit board plotter (SERIAL 1)

1	-		-	1

2	-	TXD	-	3

3	-	RXD	-	2

4	-		-	5

5	-		-	4

7	-	GND	-	7

8	-		-	20

15,17	-		-	24

20	-		-	8

24	-		-	15,17

The cable screening is connected only on the PC side (port).

The following parameters must be observed:

baud rate		9600 (default setting, others upon request)

parity			none

data bit		8

stop bit		1

hardware handshake

The PC's serial interface is initialised with the DOS command:  Mode Com1:96,N,8,1.

## 6.2.	The LPKF 91S

### 6.2.1.Command structure

The SMCU control unit interprets the HP-GL commands described below and converts them into set reactions.

The syntax:

| Symbol | Meaning |
| --- | --- |
| {...} | The contents can be repeated any number of times. |
| () | The command parameters are between these brackets. |
| [] | The content of these brackets is optional and does not have to be included. |

The SMCU expects a separator between the command parameters which is not any of the figures or capitals.  A new command can follow a parameter without a separator.  The last character in a command file which is sent must be a semicolon or a line feed symbol (0A hex.).

Unknown commands are ignored by the control unit, but their parameters can lead to unwanted plot absolute or plot relative commands.

### 6.2.2.	HP-GL standard commands

AA (x,y,a{,ß})){;}

Arc absolute

Draws an arc around the absolute coordinate (x,y) starting from the current position with the arc angle a=[degree].  With a negative arc angle a, it is drawn clockwise, but otherwise anticlockwise.

AR (x,y,a{,ß}){;}

Arc relative

Draws an arc around the relative coordinate (x,y) starting from the current position with the arc angle a=[degree].  With a negative arc angle a, it is drawn clockwise, but otherwise anticlockwise.

CI (r{,ß}){;}

Circle

Draws a complete circle around the current position with radius r.  Resolution ß is ignored as the maximum resolution is always used when drawing.

EA (x,y){;}

Edge rectangle absolute

Draws a rectangle defined by two corner points lying diagonally to each other.  The first corner point is determined by the current position and the second by the absolute coordinate (x,y).

ER (x,y){;}

Edge rectangle relative

Draws a rectangle defined by two corner points lying diagonally to each other.  The first corner point is determined by the current position and the second by the relative coordinate (x,y).

IN {;}

Initialize

Changes the control unit to the same status as after switching on.  This restores all default settings.

IW (x0,y0,x1,y1){;}

Input window

Restricts the working area on the XY axis to a window with the given corner coordinates.

OH {;}

Output hard clip limits

The control unit automatically determines its maximum travel range within the limit switches and send the coordinates determined in this way as an ASCII string in the form ("W Xmin,Ymin,Zmin,Xmax,Ymax,Zmax<cr>") to the PC.

OS {;}

Output status

The control unit sends its status line as ASCII hex ("S xxxx <cr>") to the PC.

PA (x1,y1{,...xn,yn}){;}

Plot absolute

Draws a line from the current position to the absolute coordinates listed one after the other.  The commands PU and PD can be given between the pairs of coordinates as parameters.  If additional pairs of coordinates are sent, the command word PA is not necessary.  All pairs of coordinates without a command word are taken to relate to the last PA or PR commands sent and are carried out accordingly.

PD {;}

Pen down

Lowers the tool.

PR (x1,y1{,...xn,yn}){;}

Plot relative

Draws a line from the current position to the relative coordinates listed one after the other.  The commands PU and PD can be given between the pairs of coordinates as parameters.  If additional pairs of coordinates are sent, the command word PA is not necessary.  All pairs of coordinates without a command word are taken to relate to the last PA or PR commands sent and are carried out accordingly.

PU {;}

Pen up

Lifts the tool.

VS (v{,n}){;}

Velocity select

Defines the track speed in the XY level v=[mm/s] with the tool lowered and allocates this speed to the tool with number n.

### 6.2.3.	Special Commands

All special commands begin with the symbol "!" and their syntax is otherwise like that for HP-GL standard commands.

!AS (a){;}

Acceleration set

Defines a new acceleration constant a=mm/s².  The valid value range is 10...50000.

!CC {;}

Close channel

Ends a data transmission introduced with !OC.

!CM (n){;}

Change mode

Switches between the drill (n=0) and mill (n=1) working modes.

!CT (n){;}

Command counter

Switches between echo mode (n=1) and non-echo mode (n=0).  In echo mode, the machine acknowledges every correctly completed command with the message "C<cr>".

!EM (n){;}

External motor

Switches the mill/drill motor on (n=1) or off (n=0).

!ES (n){;}

Enable stop

Release (n=1) and block (n=0) the external stop function.  After switching on, the external stop function is blocked.

!FP {;}

Full power

Switches the motors of the XYZ axis to full power.

!HP {;}

Half power

Switches the motors of the XYZ axis to half power.

!OC {;}

Open channel

Opens a direct data transmission from serial data channel SER1 to serial data channel SER2.  All characters received on data channel SER1 are sent to data channel SER2 until the character string !CC is received.

!ON (a){;}

Output nominal position

The control unit sends the nominal position (target position) of the motor axis addressed with parameter a.  The following addresses are attributed to the motor axes:

0 = X,Y,Z axis

1 = X axis

2 = Y axis

3 = Z axis

!RD (a){;}

Read port

Effects the reading of the input port with the address a=<0..15>.  The data is output via the serial interface from the which the command came, as an ASCII figure.

!RS (r){;}

Resolution set

Gives the control unit the step size (r=mm/step) of the machine.  The valid value range is 1...32000.

!TA (x,y,z){;}

Plot three-D absolute

Carries out a physical movement from the current position to the absolute coordinate (x,y,z).

!TD (t1,t2,t3,t4){;}

Time for drilling

Sets new drill times t=[ms].

!TM (t1,t2,t3,t4){;}

Time for milling

Sets new mill times t=[ms].

!TR (x,y,z){;}

Plot three-D relative

Carries out a physical movement from the current position to the relative coordinate (x,y,z).

!TS (t){;}

Time to stabilize

Sets a stabilisation time t=[ms] between the individual commands.

!TW (t){;}

Time to wait

The following command is only carried out after a wait t=[ms].

!VU (v){;}

Velocity if pen up

Defines the track speed v=[mm/s] of the movement in XY plane when the tool is raised.

!VZ (v){;}

Velocity Z axis

Defines the speed v=[mm/s] of the movement in the Z axis.

!WR (a,d{,m}){;}

Write port

Effects an output of the data word d on the port address a=<0..15>, whereby a bit mask can be given.

!ZA (z){;}

Plot Z axis absolute

Moves the Z axis from the current position to the absolute coordinate (z).

!ZR (z){;}

Plot Z axis relative

Moves the Z axis from the current position to the relative coordinate (z).

6.2.4.	Direct commands

Direct commands are special commands which, once they have been interpreted, are carried out via the command buffer.

!CB{;}

Clear buffer

Deletes all commands from the command buffer.

!GO{;}

Go on

Removes the stop command and continues the command implementation.

!RC{;}

Repeat command

Repeats the last command carried out.

!ST{;}

Stop

Interrupts command processing when the current command is completed.

The command set implemented is a subquantity of the HP-GL.  If the equipment is to be driven by any CAD system, with HP-GL commands, it must be ensured that the step size is substantially lower than with normal pen plotters.

The step size is 7.9375 mm/step (6.35 mm/800 steps).

## 6.3.	Tutorial on producing a double-sided PCB

The job to be carried out must already be loaded or prepared in BoardMaster.  For example, the data from the CircuitCAM tutorial can be used.

A further requirement is that the HOME position was programmed precisely as described above.

1.	Move to the PAUSE position using BoardMaster (Machine, PAUSE).

2.	Fix the base material using alignment pins as described in the section entitled "Securing the PCB on the machine bed".

3.	Using BoardMaster, move to the bottom left corner (-x, -y) of the base material so that the working depth limiter does not quite touch the adhesive tape.

4.	This corner must be programmed with Set Low Corner under Machine.

5.	In the same way, move to the top right corner (+x, +y) of the material and program (Material, Set High Corner).

6.	The material must be visible in BoardMaster as light grey on the dark grey machine surface.

7.	If applicable, move the project so that the graphics data is completely on the material.

8.	Select the first machining phase Drill Component.

9.	Drill data is selected with All+.  Selected data is shown brighter.

10.	Select drill mode.

11.	Switch on Auto-Motor-On (motor key on right in BoardMaster).

12.	When the motor turns, press Start.

13.	BoardMaster requests the tool needed for the machining phase by travelling to the tool change position and then switching the motor off automatically.

14.	After releasing the Allen screw, the tool can be removed from the motor shaft using the tweezers supplied.

15.	Insert the appropriate tool as far as it will go and secure with the Allen screw.

16.	Acknowledge the tool change with OK.

17.	The motor switches on and the first hole diameter of the drill phase is carried out.

18.	Once the holes of the current diameter have been made, the next tool is requested and changed in the same way.

19.	When the drill phase is complete, mill phase Component Side is selected after any feedthrough.

20.	The milling depth should now be set.  To do this, click 1Unimill in the tool combo box.  The head travels to the tool change position.  Insert the universal milling cutter and, with the arrow keys, move over the base material, but not over the project, in BoardMaster.

21.	Go into mill mode, switch on the motor, lower the head and move with the arrow keys.  Set the milling depth.  See too the section entitled "How the mechanical working depth limiter works".

22.	The mill data for the component side is now selected with ALL+.  Machining is started with Start.

23.	Then move to the pause position and turn the material.

24.	Select and carry out the Solder Side mill phase.

25.	If required, select the Cutting mill phase and machine with a contour milling cutter.

26.	Move to the pause position, remove the PCB and proceed as described in the section entitled "Cleaning the PCB".

# 7.	Tools and accessories

## 7.1.	Tools

The tools for the LPKF 91S come in two different lengths.  Tools used to machine the material surface (milling and engraving) are 30 mm long, and tools used for drilling or contour milling are 32 mm long.  The following tools are available.

µ §

LPKF tools for circuit board plotters

1.	Universal LPKF milling cutter and drill, for milling isolation channels and for front panel engraving from 0.2 to 0.5 mm (depending on depth set), 30 mm long.

2.	Film milling cutter, 0.3 mm, for making typons, 30 mm long.

3.	Contour miller for engraving wider isolation channels.

4.	Double chamfered cutter (0.8; 1; 2 mm) for front panel engraving, 30 mm long.

5.	Spiral drill, various diameters for drilling in PCB material, 32 mm long.

6.	Contour milling cutter, for milling out gaps in PCB material, 32 mm long.

The tools must in all cases be inserted in the tool fixture as far as they will go.  Otherwise the working depth will be incorrect.  In certain circumstances, this can even result in damage to the machine base plate.

Tweezers are supplied for tool insertion and removal.

## 7.2.	Pen holder (optional)

With the pen holder, the circuit board plotter can be used as a pen plotter.  The pen holder is fitted as follows:

1.	Insert the aluminium part of the pen holder in the hole in the motor support plate and secure it with the Allen screw supplied.

2.	Screw the ink pen in the brass socket.

3.	Now insert the brass socket with the ink pen in the aluminium bracket.

4.	Set the working depth limiter so that the ink pen just comes in contact with the paper.

# 8.	Milling and drilling with the LPKF 91S.

## 8.1.	Securing the PCB on the machine bed

Make overlapping alignment holes (3.05 mm in diameter) in the base material and drilling base in the X direction.  This can be done using any stand drilling machine.  Remember that the diameter is reduced by galvanic feedthrough.

Move the mill/drill head to the PAUSE position.  Insert alignment pins in the front and rear two-pin strips but, even so, check that the front two-pin strip is pushed forwards against the reference pin in the machine groove.

Position the pre-drilled base material and drilling base over the alignment pins.  The base material format should be chosen such that the two-pin slide does not have to be pushed more than 10 mm as the two-pin system can otherwise become less accurate.

Now secure the base material in position with adhesive tape (Tesakrepp 5250 is recommended) on all sides.  This prevents the corners of the PCB turning upwards.

µ §Securing the PCB

1.	Machine bed (aluminium base plate)

2.	Alignment pins, 3 mm in diameter

3.	Base material, approx. 1.6 mm thick

4.	Drilling base, 2 mm thick

5.	Adhesive tape

6.	Two-pin strip

The alignment pins hold the PCB in position.  This is essential particularly for contour milling.  They are also however the reference when turning double-sided PCBs.  The adhesive tape holds the PCB down at the edges.

It is important that there is no dirt (adhesive tape remains, drilling or milling chips) between the individual layers so that the base material can be laid absolutely flat.  Small particles under the base material would adversely affect milling depth uniformity.

## 8.2.	How the mechanical working depth limiter works

When milling isolation channels in often easily bent PCB materials, it is extremely important to keep a constant milling depth.  With the LPKF 91S, the mechanical working depth limiter does this.

This provides the following benefits:

1.	The material is scanned.  The working depth limiter follows easily bent PCBs.

2.	The material is held down within certain limits by the working depth limiter.

The head is lowered with a lifting magnet and raised with a spring.

µ §

The LPKF 91S working depth limiter

1.	Holding plate

2.	Knurled nut used to set milling depth

3.	Holding block for working depth limiter

4.	Suction nozzle

5.	Tool fixture with Allen screw

6.	Scanning pins in the working depth limiter

Milling depth is set by adjusting the knurled nut (2) on the working depth limiter.  When the wheel is turned clockwise, the milling depth is increased, while it is reduced when turned anticlockwise.  The milling depth is altered by about 20 mm per step.  Turning clockwise is, particularly when the head is lowered, easier than anticlockwise.

## 8.3.	Functional elements on the mill/drill head

µ §

Mill/drill head, view from front

1.	Shock absorber

2.	Lifting magnet

3.	Adjustment, bottom head stop (do not move)

4.	Working depth limiter

5.	Motor cover

6.	Fixing screw for motor cover

7.	Holding bracket for DC motor

8.	DC motor

9.	Setting screw for knurled nut (do not move)

10.	Fixing screw for working depth limiter

11.	Base plate for DC motor

µ §

Mill/drill head, view from above

1.	Adjusting nut for head stop, top

2.	Lifting magnet

3.	Shock absorber

4.	Z guide

5.	Hole for pen holder

6.	Fixing screw for working depth limiter

7.	DC motor

8.	Base plate for DC motor

9.	Bracket for DC motor

## 8.4.	Drilling

µ §

PCBs are drilled with special PCB drills.  It is important always to lower the head at a constant speed.  This is achieved by setting the tool as close as possible to the material to be drilled.  A setting nut can be used to make this adjustment.  An excessive lowering speed can result in burrs, particularly where holes have a small diameter.  Only one PCB can be drilled at a time.  It is not possible to stack PCBs one on top of the other.  No drill cover plate is needed.  All drills are 32 mm long.

## 8.5.	Isolation milling

µ §

Before milling, it must be ensured that sufficiently sharp LPKF universal milling cutters (30 mm long) are used.  The milling width is set to between 0.2 and 0.5 mm depending on component thickness.

It is advisable to degrease the base material with cleaner spray before machining (degreaser for electronic components), so that milling dust can be removed more easily by the extractor.

The milling depth must be set as large (deep) as possible in all cases.  If only the extreme milling cutter tip (<0.2 mm isolation) is used, the tool wears more quickly than with deeper drilling.

µ §

Milled channel with universal milling cutter and micro-milling cutter

For the advantages and disadvantages of various materials, see the section entitled "Base material".

After machining, the PCB has to be cleaned.  This can be carried out either in a brush machine or alternatively manually with plate cleaners (LPKF accessories).  In either case, the PCB must be rinsed thoroughly with water to remove any copper dust produced by brushing.

After rinsing, the PCB must be dried thoroughly (air drier) and then protected against oxidation by a solder varnish.

## 8.6.	Contour milling in PCB material

µ §

The speed of movement has to be lowered for contour milling.  This may differ from material to material.  Only use special contour milling cutters (32 mm long):  if possible 1 or 2 mm.

The 1 mm contour milling cutter should only be used for internal holes (smaller internal radius).   It breaks relatively easily so set the feed speed to minimum.

The 2 mm contour milling cutter is substantially more robust, but does not remove as much material as the 3 mm milling cutter.  Many LPKF software postprocessors are calculated for the 2 mm milling cutter (e.g. CUTTING).

With contour milling using a 3 mm milling cutter, a large quantity of material is removed.  Moreover, the DC motor can be overloaded if the speed of movement is too fast.

## 8.7.	Milling wide isolation channels

µ §

Wide isolation channels can be made with a 30 mm long contour milling cutter.  Various diameters are available.

The milling depth must be set such that when the copper is removed, only minimal burring results.  The 3 mm contour milling cutter is particularly well suited for milling very wide isolation channels (VDE regulations).

## 8.8.	Front plate engraving

µ §

For engraving, set the speed of movement as appropriate for the engraving depth and the material.  Use an LPKF universal milling cutter or LPKF double chamfered cutter.

Use extractor when engraving too.

Warning:  aluminium front panels cannot be milled through with the LPKF 91S circuit board plotter.

## 8.9.	Milling layout films

µ §

Secure the film base (sheet of perspex or glass) on the machine table with Tesaband (adhesive tape).

Lay the film material on the base with the coated (matt) side facing upwards.

Now smooth the film material firmly and level on the film support until the air has been completely expelled.

Now stick down the film on all four sides with transparent adhesive tape (which must not stretch) to form an air seal.  There must not be any air bubbles between the film and the base.  The milling depth can now be set at the film edge.  It can be checked by milling a frame around the film area (manual movement).

The speed of movement should be reduced to about 15 mm/sec.

Switch on extractor, but only to half power, by extracting "secondary air" on the suction nozzle or, if there is one, reducing the power on the electronic extractor control.  The film milling program can now be started.

Film material comes in DIN A3 and A4 formats (special sizes available upon request).

Important:  the film coating is easily scratched and is water soluble, so do not let it come into contact with water.  The film can be recopied with a colouring device upon request.

### 8.9.1.Correction agents

Gravure films can be corrected with Duroscal Korrekturschicht rot, which is easy to use.  Paint in the problem areas with it, leave to dry for about 5 minutes and then further process the gravure film in the normal way.

The corrected areas can only have their coating removed with a quick acting coating remover.

Follow the safety instructions on the correction agent container.

### 8.9.2.Colouring milled films

For milled negative films, there is Duroscal Einfärber schwarz, with which positives can be made.  The fully milled gravure film is coloured with Duroscal Einfärber by pouring the liquid onto a SAFIR pad and spreading it evenly over the whole film.  Excess colour must be wiped off with cellulose wadding.  The stained film must be held against the light to check it and to find poorly covered areas so that they can be recoloured.  Then immediately remove the coating.  The colour must not dry.  Important:  the liquid must not get onto the back of the film as it cannot then be removed from it.  The colour is also difficult to remove from other surfaces (fabrics, skin) (wear apron and rubber gloves).

Follow the safety instructions on the colour container.

### 8.9.3.Coating removal

When removing coatings, only treat one film per container to prevent any damage of the second film by contact between the back of it and a surface wet with colour.

There are two options when it comes to coating removal:

1.	Quick coating removal.  Pour the quick-acting coating remover onto the film and immediately wipe the dissolved protective coating with cellulose applying slight pressure.  Then dry the film with cellulose wadding, blotting paper or a cloth.

Follow the safety instructions on the quick-acting coating remover container.

2.	Removal with water.  For this method, the film must be placed in a bowl of hand-hot water (to which a small amount of washing-up liquid has been added).  The protective layer dissolves after about 30 minutes.  Any coating remains must be removed with a fine hand brush.  This method is cheaper and more environmentally friendly.  The film can be left in the water bath as long as you want.

## 8.10.	Base material

Technically, it is possible to process all base materials.

However, the most basic phenol resin qualities (FR 2) can adversely affect milling quality.

Glass fibre reinforced epoxy material (FR 4 or G 10) can be a health hazard due to the milling dust produced (allergies, risk of cancer).  Tool service life is also substantially reduced.

We recommend epoxy material without glass fibre (FR 3).  With top milling quality and a high tool service life, no disadvantages with regard to FR 4 are known other than a slightly reduced mechanical bearing capacity (breakage).  This drawback should not be of any significant importance for prototype plates unless particularly heavy components are to be mounted.

Normally, a total thickness of 1.5 mm and a Cu thickness of 35 mm are used.  With 17 mm material even fine milling channels can be engraved at a higher packing density.  For galvanically through-hole-plated, double-sided PCBs, 5 - 17 mm Cu thickness is used to prevent an excessively thick copper layer after galvanic copper application.

With 70 mm material, compromises must be made when setting milling depth.  I.e. a deeper milling depth results in wider milling channels of 0.5 to 0.7 mm.

Special base materials with a thicker copper layer of up to 300 mm can no longer be machined with the LPKF universal milling cutter.  Contour milling cutters or special tools are needed for this.  In these cases, we would ask that you consult us and send sample material so that tests can be carried out if appropriate.

Teflon materials can be machined, but the following points must be borne in mind:

1.	As the material is very soft and therefore often extremely uneven, a constant milling width often cannot be maintained, so the material should be smoothed first.

2.	Because the material is so soft, it is not possible to mill such fine isolation tracks as in epoxy material.

The following material has proved particularly successful as material for HF and microwave applications:  RT/Duroid types 5870, 0310 diel and 001 10 oz electro-deposited copper 2 sides.

A drilling base is indispensable for all machining processes on PCB material.  With it, PCBs can be drilled through without damaging the machine.  The drilling base can be made simply of cardboard and should be 2 mm thick.

## 8.11.	Cleaning the PCB

Before components are mounted, the finished PCB must be thoroughly cleaned.  This can be carried out manually or in PCB brushing machines.

If cleaning by hand, the PCB is placed on a flat support.  The board is brushed in the direction of the conductor paths with wet board cleaner (e.g. LPKF board cleaner PAD).  The purpose of the brushing is on the one hand to remove the layer of oxidation and on the other to remove swarf in the isolation channels.  After brushing, the PCB must be free of any metal particles.  From now on, the board should only be held by its edges.  The board should now be rinsed under running water and then dried with an air drier.  Important:  never use compressed air as the oil particles it contains can cause problems later on.  After drying, both sides of the board are coated with solderable lacquer.

## 8.12.	Practical tips

	Set the milling depth such that engraving is too deep rather than too shallow.  Insufficient depth when engraving promotes milling tool wear.

	Misalignment of the solder and component sides occurs where the HOME position is not accurately programmed.

	There can be a number of causes of uneven milling width (depth).  It is important that the machine bed is really clean.  Residues of adhesive tape or such like can adversely affect milling depth quite considerably.  Also, milling swarf between the machine bed, drilling support and PCB can reduce precision.  Greatly distorted materials bend such that the sag shows underneath;  in this case, secure the edges well with adhesive tape.  Another important point for precise milling depth is the removal of milling and drilling chips.

	In film milling, milled channels are uneven if there is still air under the film.  If the film is secured with elastic adhesive tape, waves may form in the film after a while.  It is particularly important here that the milling base should be level (LPKF film engraving base).

	Burrs are produced when milling films either because the tool is blunt or milling was too deep.

	Hooks can occur between the milling channels if the incorrect milling sequence was followed, in particular with circles.  If a circle is to be milled with a tool which rotates clockwise, fine hooks can arise between the copper areas if the milling tracks should overlap.  The reason for this is that the cutting speed on the outer edges is reduced.  The solution lies in selecting the right milling direction.  When isolating conductor paths with LPKF isolate, the solder side should be mirrored before isolating as the isolation algorithm itself works in a clockwise direction.  The standard postprocessing is already designed for this.

	Milling burrs can be caused by blunt tools or incorrect speeds of movement.  If possible with the structure to be milled, deeper settings can be the answer.  Otherwise, change the tool.

	Drilling burrs occur either because the tool is blunt or the head lowering speed is excessive.  In the first case, change the tool.  In the second case, the tool height over the material must be reduced.

	Burrs when contour milling or cut edges which are not clean occur either due to a blunt tool or incorrect advance speed.

	With some materials, the colour of the milled channel gives some indication of the state of the tool.  With epoxy materials, dark isolation paths indicate a sharp tool, while lighter ones indicate a blunter tool.

	Drill deflection occurs in particular with thin tools which are no longer absolutely sharp.  However, this also depends on the surface structure of the material.  If, for example the glass fibre structure of FR4 materials penetrates the copper, drill deflection cannot be avoided even with a sharp tool.  For materials with additional, removable copper film (FR4 material with 18 mm or 9 mm Cu coating), drill deflection is very slight.

	Vitrification of the drilled hole occurs where the drill stays in the hole too long once the hole has been made.  These holes then cause problems at feedthrough stage.  Reduce drilling times as appropriate.

	Drills break where the drilling base has already been used a number of times.  The drilling base should be changed for every new PCB.  It is not really possible to prevent breakage if a drill comes into contact with the edge of an existing hole in the base.  Broken tools must be removed from the PCB and the drilling base.  Drills can also break if the tool is too high above the material.

# 9.	Appendix

## 9.1.	Maintenance

Keep spindles clean.  In rooms with a high air humidity, wipe down with a lightly oiled cloth from time to time.  Keep transport spindles oiled and clean.

Do not oil bearings!

Strip and clean working depth limiters at regular intervals.  To do this, first switch all the equipment off, then proceed as follows:

1.	Release Allen screw with which the holding block is fixed on the motor retaining plate.

2.	Unscrew the two M5 Allen screws which hold the mill/drill head on the bearing cross plate.

3.	Unscrew the aluminium part of the working depth limiter.

4.	Clean both parts of the working depth limiter with the brush provided.

5.	Important:  only lightly oil the thread of the working depth limiter.

6.	After cleaning, the working depth limiter is refitted, following the procedure in reverse.

## 9.2.	Serial port SERIAL1

The serial data channel SERIAL 1 is an RS 232-C standard interface with galvanic decoupling and is used for communication between the control unit and a PC.  The DCD signal input on the control means that the control unit can be STOPPED quickly when the signal level changes from low to high.  In this way, it is possible to stop the machine directly from the PC by activating the DTR signal.  The SERIAL 1 transmission speed is programmed with DIL switches 1 and 2.

Switch 1	Switch 2	Baud rate

OFF		OFF		4800 baud

ON		OFF		9200 baud

OFF		ON		19400 baud

ON		ON

SERIAL 1 settings:

Pin		Signal	Meaning

2		TXD		Transmit data

3		RXD		Receive data

4		RTS		Request to send

5		CTS		Clear to send

7		GND-I	Ground (isolated)

8		DCD		Data carrier detect

20		DTR		Data terminal ready

All signals are galvanically decoupled;  all contacts not mentioned are not used.

## 9.3.	Serial port SERIAL2

The serial data channel SERIAL 2 is an RS 232-C standard interface and can be used for communication between the control unit and another system.  The transmission speed of SERIAL 2 is programmed with DIL switches 3 and 4.

Switch 3	Switch 4	Baud rate

OFF		OFF		4600 baud

ON		OFF		9200 baud

OFF		ON		19400 baud

ON		ON		-

SERIAL 2 settings:

PIN				Signal	Meaning

3		6		TXD  		Transmit data

2		4		RXD  	Receive data

7		5		RTS  		Request to send

8		7		CTS  		Clear to send

5		10		GND  	Ground

All contacts not mentioned are not used.

## 9.4.	Motor connection

The stepping motors are connected per axis via a 15-pole SUB-D plug.  If the limit switch is actuated, any further axis movement in the direction of the limit switch is immediately blocked.  The position of the sockets is described in the section entitled "LPKF 91S displays and connections".

Important:  never confuse the motor and the mill/drill head cables!

## 9.5.	Mill head connection

The mill/drill head is connected to the control unit with a 15-pole socket.  The socket position is described in the section entitled "LPKF 91S displays and connections".

Important:  never confuse the motor and the mill/drill head cables!

## 9.6.	EPROMs

The software for the control unit is in 2 EPROMs.  If the EPROMs need to be replaced (updates), the LPKF 91S has to be partially dismantled, so follow the procedure described in the section entitled "Fuses" and change the EPROMs, ensuring that the EPROMs are correctly aligned and positioned.

## 9.7.	Fuses

In the LPKF 91S, the primary and secondary voltages are fused.  The main fuses (primary) are in the mains connection of the control unit and are accessible from the outside.

The secondary fuses are inside the control unit.  The control unit must first be removed from the mechanical part to replace the fuses.  To do this, loosen screws 1, 2, 3 and 4 and remove the cable connections between the mechanical part and the control unit.

µ §

Position of screws for removing the electronics from the mechanical part of an LPKF 91S

Remove the cover with the x limit switches.  The control unit thus released is constructed as follows:

µ §

View of LPKF 91S PCB

1.	Mains filter

2.	Transformer

3.	Transformer connection

4.	Power pack for mill/drill head

5.	Power pack, Z axis (free)

6.	Power pack, Y axis (free)

7.	Power pack, X axis (free)

8.	Processor section

9.	DIL switches for baud rate setting (with default setting)

10.	Power packs

11.	Processor

12.	EPROM L

13.	EPROM H

701-705	Various fuses (see below)

The secondary current circuits are protected with fine-wire fuses as follows:

| Fuse | Type | Power circuit |
| --- | --- | --- |
| F701 (not used) | 3.15 A | Power high level |
| F702 | 3.15 A | Power high level |
| F703 | 3.15 A | +24 V at I/O interface |
| F704 | 2.00 A | +5 V and VREF |
| F705 | 0.50 A | RS 232 C/SER 1 |

The units are mounted in the opposite sequence to that described above for dismantling.

Important:  never confuse the motor and the mill/drill head cables!

Before switching on, always check that the head and motor cables are in the correct place.

The fuses in the primary current circuit are in the mains switch, which also contains the mains filter and the mains commutator (e.g. 230 V/115 V).  The fuses should be semi-time lag:

230 V	fused with 3.15 A

115 V	fused with 6.3 A.

## 9.8.	List of LPKF tools available

Tools shaded grey cannot be used for the LPKF 91S.

| Name |  | Diameter |  | Length |  |  |  | Delivery |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | in mm |  | 30 mm |  | 32 mm |  | L = in stock |  |  |
| Universal milling cutter |  | 0.2 - 0.7 |  | * |  |  |  | L |  |  |
| Film milling cutter |  | 0.3 |  | * |  |  |  | L |  |  |
| Contour milling cutters |  | 1.0 |  | * |  | * |  | L |  |  |
|  |  | 1.5 |  | * |  | * |  | L |  |  |
|  |  | 2.0 |  | * |  | * |  | L |  |  |
|  |  | 2.5 |  | * |  | * |  | L |  |  |
|  |  | 3.0 |  | * |  | * |  | L |  |  |
| Double chamfered cutters |  | 0.8 |  | * |  |  |  | L |  |  |
|  |  | 1.0 |  | * |  |  |  | L |  |  |
|  |  | 2.0 |  | * |  |  |  | L |  |  |
|  |  | 3.0 |  | * |  |  |  | L |  |  |
| Drills |  | 0.7 |  | * |  | * |  | L |  |  |
|  |  | 0.75 |  | * |  | * |  |  |  |  |
|  |  | 0.8 |  | * |  | * |  | L |  |  |
|  |  | 0.85 |  | * |  | * |  | L |  |  |
|  |  | 0.9 |  | * |  | * |  | L |  |  |
|  |  | 0.95 |  | * |  | * |  |  |  |  |
|  |  | 1.0 |  | * |  | * |  | L |  |  |
|  |  | 1.05 |  | * |  | * |  | L |  |  |
|  |  | 1.1 |  | * |  | * |  | L |  |  |
|  |  | 1.15 |  | * |  | * |  |  |  |  |
|  |  | 1.2 |  | * |  | * |  | L |  |  |
|  |  | 1.25 |  | * |  | * |  | L |  |  |
|  |  | 1.3 |  | * |  | * |  | L |  |  |
|  |  | 1.35 |  | * |  | * |  |  |  |  |
|  |  |  | 1.4 |  | * |  | * |  | L |  |
|  |  |  | 1.45 |  | * |  | * |  |  |  |
|  |  |  | 1.5 |  | * |  | * |  | L |  |
|  |  |  | 1.55 |  | * |  | * |  |  |  |
|  |  |  | 1.6 |  | * |  | * |  | L |  |
|  |  |  | 1.65 |  | * |  | * |  |  |  |
|  |  |  | 1.7 |  | * |  | * |  | L |  |
|  |  |  | 1.75 |  | * |  | * |  |  |  |
|  |  |  | 1.8 |  | * |  | * |  | L |  |
|  |  |  | 1.85 |  | * |  | * |  |  |  |
|  |  |  | 1.9 |  | * |  | * |  | L |  |
|  |  |  | 1.95 |  | * |  | * |  |  |  |
|  |  |  | 2.0 |  | * |  | * |  | L |  |
|  |  |  | 2.05 |  | * |  | * |  |  |  |
|  |  |  | 2.1 |  | * |  | * |  | L |  |
|  |  |  | 2.15 |  | * |  | * |  |  |  |
|  |  |  | 2.2 |  | * |  | * |  | L |  |
|  |  |  | 2.25 |  | * |  | * |  |  |  |
|  |  |  | 2.3 |  | * |  | * |  | L |  |
|  |  |  | 2.35 |  | * |  | * |  |  |  |
|  |  |  | 2.4 |  | * |  | * |  | L |  |
|  |  |  | 2.45 |  | * |  | * |  | L |  |
|  |  |  | 2.5 |  | * |  | * |  | L |  |
|  |  |  | 2.55 |  | * |  | * |  |  |  |
|  |  |  | 2.6 |  | * |  | * |  | L |  |
|  |  |  | 2.65 |  | * |  | * |  |  |  |
|  |  |  | 2.7 |  | * |  | * |  | L |  |
|  |  |  | 2.75 |  | * |  | * |  |  |  |
|  |  |  | 2.8 |  | * |  | * |  | L |  |
|  |  |  | 2.85 |  | * |  | * |  |  |  |
|  |  |  | 2.9 |  | * |  | * |  | L |  |
|  |  |  | 2.95 |  | * |  | * |  |  |  |
|  |  |  | 3.0 |  | * |  | * |  | L |  |
|  |  |  | 3.05 |  | * |  | * |  |  |  |

## 9.9. Spare parts for the LPKF 91S

Paßstifte	M-00-013

Paßlochstreifen	M-89-048a

DC Motor (ohne Werkzeugaufnahme)	M-89-001

Werkzeugaufnahme	M-89-006

Arbeitstiefenbegrenzer	M-89-003

Spindelmutter	M-89-028

Fixierschraube für Werkzeugaufnahme	M-89-005

Absaugschlauch	M-89-018

Stoßdämpfer	M-91-001

Bei Ersatzteilbestellungen, bzw. Anfragen bitte immer Maschienennummer mit angeben. Die Nummer ist auf der Grundplatte eingestanzt.